Teltonika RUTX08 Router Setup

Initial Setup

Hardware Setup:

- PC should be connected to power and internet

- Connect USB wire from PC to Beagle Bone Black

  - *blue light should flicker on BBB*

- Connect Ethernet wire from BBB to LAN1

- Connect RUTX08 Power cable into 24V

  - *PWR LED should be steady green, LAN1 port LED should be steady orange*

Install Python Libraries on PC

pip install paramiko scp ping3 --trusted-host pypi.org --trusted-host files.pythonhosted.org --trusted-host pypi.python.org

Get into Beagle Bone Black:

ssh raj@192.168.7.2

Set static IP from BBB to Router:

sudo ip addr flush dev eth0

*asked for sudo password* [Jetway]

sudo ip addr add local 192.168.1.2/24 dev eth0

*asked for sudo password* [Jetway]

sudo ip link set eth0 up

ping 192.168.1.1

*should be receiving packets*

can use ip addr show eth0 to show current IP(s) setup

Firmware

Download Firmware:

go to https://wiki.teltonika-networks.com/view/RUTX08_Firmware_Downloads and download firmware

or wget –P /home/raj/ [https addr of firmware.bin file]    only possible if BBB is connected to internet

Perform Checksum:

Get-FileHash -Path [path to firmware download on pc]

Verify the string under the hash column is the same as the string on the site below for the firmware used

RUTX Firmware checksum list - Teltonika Networks Wiki

Transfer Firmware from PC to BBB to Router

scp [path to file on pc] raj@192.168.7.2:/home/raj/

*enter BBB password*

ssh raj@192.168.7.2

*enter BBB password*

scp /home/raj/[firmware.bin] root@192.168.1.1:/tmp/

*enter Router password*

Flash Firmware onto Router

ssh root@192.168.1.1

*enter Router password*

sysupgrade /tmp/[firmware.bin]

*auto reboots* *will have to add static IP again*

Change Temp Password

passwd

*enter new password twice*

Min Pass Length: 8

Digits: at least 1 number

Characters: at least 1 lower, 1 upper, and 1 special

Router Configuration

Upload Own Config File:

scp [path to file on pc] raj@192.168.7.2:/home/raj/

*enter BBB password*

ssh raj@192.168.7.2

*enter BBB password*

scp /home/raj/[firmware.bin] root@192.168.1.1:/tmp/

*enter Router password*

ssh root@192.168.1.1

*enter password*

Backup Current Config File:

sysupgrade -b /tmp/backup_before_change.tar.gz

scp root@192.168.1.1:/tmp/backup_before_change.tar.gz /home/raj

To restore: restore_config /tmp/backup_before_change.tar.gz

Backup of just the network config file: cp /etc/config/network /etc/config/network.bak

Update Current Config File:

cd /etc/config/network

cd /etc/config/firewall

use uci set commands to change network configurations

uci commit

reboot

OR

cp /tmp/[network config file name] /etc/config/network

cp /tmp/[firewall config file name] /etc/config/firewall

uci commit network

reboot